National Textile University, Faisalabad

Department of Computer Science

| Name | Malaika Batool |
| --- | --- |
| Class | BSCS |
| Section | 5th-A |
| Registration No | 23-NTU-CS-1044 |
| Course Name | EMBEDED IOT |
| Submitted To | SIR NASIR |
| Submission Date | 26/10/2025 |

## 1. Purpose

The purpose of this project is to implement an LED mode controller using an ESP32 (or compatible board) where:

- There are three LEDs connected to three GPIO pins.

- Two push-buttons (Button A and Button B) control the operational mode.

- An OLED display shows the current mode name.

- The system supports four distinct modes:

  - Both LEDs OFF

  - Alternate Blink (LED2 & LED3)

  - Both ON

  - Fade (LED1 only)

- Button A cycles through the modes.

- Button B resets the mode back to 0 (both OFF).
In short, it demonstrates GPIO control, digital input (buttons), analog output (PWM fade), and I²C OLED display usage.

## 2. Hardware Design / Pin Assignment

### Components

- Microcontroller board: typically an ESP32 (since GPIO numbers match)

- Three LEDs (with appropriate current-limiting resistors)

- Two push-buttons (configured with internal pull-ups)

- An OLED display (SSD1306) connected via I²C (SDA & SCL)

### Pin assignment

- LED1 → GPIO 16 (used for the Fade mode)

- LED2 → GPIO 17 (used for blinking)

- LED3 → GPIO 18 (used for blinking or ON mode)

- Button A → GPIO 14 (cycles modes)

- Button B → GPIO 27 (resets mode)

- OLED display:

  - SDA → GPIO 21

  - SCL → GPIO 22

  - I²C address: 0x3C

### Wiring notes

- Buttons use INPUT_PULLUP mode: so wiring one side of button to the pin, the other side to ground. When pressed, pin reads LOW.

- LEDs should have resistors (e.g., ~220 Ω) in series to limit current.

- OLED requires proper I²C connections and power (typically 3.3 V or 5 V depending on board/OLED module).

- Ensure that the pin assignments are compatible with the board and that no conflicts exist with other peripherals.

## 3. Software Design / Code Explanation

Below is an annotated walkthrough of the code.

/*

=====================================================

Title   : Task A — LED Mode Controller (Final 3 LEDs)

Author  : Malaika batool

Reg No. : 23-NTU-CS-1044

Date    : 26/10/2025

Description:

Button A (GPIO14): Cycles LED modes

1. Both OFF

2. Alternate Blink

3. Both ON

4. Fade (LED1 only)

Button B (GPIO27): Resets mode

OLED (GPIO21 SDA, 22 SCL): Displays mode info

=====================================================

*/

### Libraries and OLED setup

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_ADDR 0x3C

#define SDA_PIN 21

#define SCL_PIN 22

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

- We include the Wire library for I²C communication.

- Adafruit_GFX and Adafruit_SSD1306 libraries are used for the OLED display.

- Screen size defined for 128×64.

- I²C address set to 0x3C (common for SSD1306).

- We initialize display object with the dimensions and I²C interface.

### Pin definitions and variables

#define LED1 16  // fade

#define LED2 17  // blink

#define LED3 18  // on/off

#define BUTTON_A 14

#define BUTTON_B 27

int mode = 0;

unsigned long lastPress = 0;

- LED1 is assigned to GPIO 16, LED2 to 17, LED3 to 18.

- Buttons assigned to 14 (A) and 27 (B).

- mode keeps track of the current mode (0,1,2,3).

- lastPress tracks the time of the last button press to implement debouncing (simple 300 ms window).

### Displaying the mode

void showMode() {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 10);

display.print("Mode: ");

if (mode == 0) display.println("BOTH OFF");

if (mode == 1) display.println("ALT BLINK");

if (mode == 2) display.println("BOTH ON");

if (mode == 3) display.println("FADE");

display.display();

}

- Clears the OLED screen, sets text size and color.

- Writes “Mode: ” then prints the current mode label.

- Finally calls display.display() to push buffer to screen.

- Makes it user-friendly by showing which mode is active.

### Setup routine

void setup() {

Wire.begin(SDA_PIN, SCL_PIN);

display.begin(SSD1306_SWITCHCAPVCC, OLED_ADDR);

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(LED3, OUTPUT);

pinMode(BUTTON_A, INPUT_PULLUP);

pinMode(BUTTON_B, INPUT_PULLUP);

showMode();

}

- Begins I²C on GPIO 21 (SDA) and GPIO 22 (SCL).

- Initializes the OLED display with display.begin().

- Sets LEDs as OUTPUT and buttons as INPUT_PULLUP.

- Calls showMode() once to display initial mode (mode = 0).

### Main loop

void loop() {

if (digitalRead(BUTTON_A) == LOW && millis() - lastPress > 300) {

mode = (mode + 1) % 4;

showMode();

lastPress = millis();

}

if (digitalRead(BUTTON_B) == LOW && millis() - lastPress > 300) {

mode = 0;

showMode();

lastPress = millis();

}

static int fade = 0;

static int dir = 1;

switch (mode) {

case 0: // both off

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

digitalWrite(LED3, LOW);

break;

case 1: // alternate blink

digitalWrite(LED2, millis() / 500 % 2);

digitalWrite(LED3, !digitalRead(LED2));

digitalWrite(LED1, LOW);

break;

case 2: // both on

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

digitalWrite(LED3, HIGH);

break;

case 3: // fade on LED1

fade += dir * 10;

if (fade <= 0 || fade >= 255) dir = -dir;

analogWrite(LED1, fade);

digitalWrite(LED2, LOW);

digitalWrite(LED3, LOW);

delay(10);

break;

}

}

#### Button handling

- If Button A is pressed (pin reads LOW) and more than 300 ms since last press, mode increments (with wrap via %4).

- If Button B is pressed (and time condition) then mode resets to 0.

- After change, showMode() runs and lastPress updated.

#### Mode operations

- Mode 0 (BOTH OFF): All LEDs set LOW → off.

- Mode 1 (ALT BLINK):

  - LED2 toggles every 500 ms: millis()/500 %2.

  - LED3 is the inverse of LED2.

  - LED1 remains off.

- Mode 2 (BOTH ON): All three LEDs set HIGH → constant on.

- Mode 3 (FADE):

  - A static fade value moves up or down (by dir * 10) each loop iteration.

  - When fade hits 0 or 255, dir flips to reverse ramp.

  - analogWrite(LED1, fade): LED1 fades in/out.

  - LED2 and LED3 remain off.

  - A small delay(10) adds pace to fading.

## 4. Testing & Validation

### Initial check

- On startup, the OLED should show “Mode: BOTH OFF”.

- None of the LEDs should light.

### Button A functionality

- Press Button A: OLED should update to “Mode: ALT BLINK” and LED2/LED3 alternate blinking every ~500 ms. LED1 remains off.

- Press again: “Mode: BOTH ON”, all LEDs on.

- Press again: “Mode: FADE”, only LED1 pulses in brightness, LED2/LED3 off.

- Press again: wraps back to “Mode: BOTH OFF”.

### Button B functionality

- At any mode, pressing Button B resets to Mode 0 (BOTH OFF) immediately, with OLED update and LEDs off.

### Edge cases & robustness

- Buttons are debounced via simple 300 ms time check. Verify that rapid presses don’t cause mode skipping.

- Ensure the fade loop executes smoothly, without blocking the button detection for too long (the small delay helps).

- Check LED brightness and wiring—if too dim or too bright, adjust resistor values.

- Ensure OLED I²C works reliably: display initial text, then updates on mode changes.

### Simulation on Wokwi

- Run the simulation and verify that the correct GPIO pins correspond to the LEDs/buttons as assigned.

- Watch console and visuals: On the simulator you should see LED states change, and OLED screen updating.

## 5. Usage Guide

- Power up the board and connect the hardware as per pin assignments.

- On startup you’ll see the display show “Mode: BOTH OFF”.

- To cycle through modes, press Button A each time.

- To reset to mode 0 at any time, press Button B.

- Modes and behavior explained:

  - Mode 0: All LEDs off → energy-saving/off state.

  - Mode 1: LED2 & LED3 blink alternately → visual indicator mode.

  - Mode 2: All LEDs ON → constant on status.

  - Mode 3: LED1 fades in/out → eye-catching or decorative mode.

- The OLED always displays the current mode label for clarity.

## 6. Extensions & Improvements

Here are some ideas for further enhancement:

- Add more modes (e.g., chasing LEDs, random blink, PWM on multiple LEDs).

- Use interrupts for button presses instead of polling for more responsive behavior.

- Add long-press detection for additional actions (e.g., toggle speed, brightness).

- Store the last mode in non-volatile memory (EEPROM) so upon restart it resumes previous mode.

- Add speed/pattern adjustable via potentiometer or second set of buttons.

- Expand the OLED display to show more status (e.g., mode number, brightness value, timer).

- Integrate a real-time clock or sensor to trigger mode changes automatically based on time or environment.

## 7. Summary

This project successfully demonstrates a compact controller for LEDs with multiple modes, user interaction via buttons, real-time feedback via an OLED display, and usage of both digital and analog (PWM) pins. The structure is clear: initialisation, input handling, mode logic, output control. It is well-suited as a teaching example of embedded systems design on an ESP32-type board, combining input, output, display and control logic.

SCREENSHOT:

## TASK B

### 1. Objective

To use one button on an ESP-board such that:

- A short press (< 1.5 s) toggles an LED.

- A long press (> 1.5 s) triggers a buzzer tone.

- An OLED display shows the event (“Short press: LED ON”, “Long press detected”, etc.).

### 2. Hardware Setup

| Component | Pin | Function |
| --- | --- | --- |
| Button | GPIO 14 | Detect press and release |
| LED | GPIO 16 | Toggle ON/OFF on short press |
| Buzzer | GPIO 25 | Sound on long press |
| OLED Display | SDA → GPIO 21, SCL → GPIO 22 | Displays current event |

Button is wired using internal pull-up (INPUT_PULLUP).

### 3. Software Overview

- Uses Adafruit_GFX + Adafruit_SSD1306 for the OLED.

- In loop(), reads the button state.

  - On press (state goes LOW): capture pressStart = millis().

  - On release (state goes HIGH): compute pressDuration = millis() - pressStart.

    - If pressDuration < 1500 ms: it’s a short press → toggle LED state, update OLED message.

    - Else: it’s a long press → update OLED message + call beep() to drive buzzer.

- The beep() routine toggles the buzzer pin at a frequency (~1000 Hz) for ~400 ms.

- Debouncing via a small delay(10) and tracking previous button state.

### 4. Testing & Validation

- On startup the OLED should display: “Ready… press button”.

- Short press the button (quick tap): LED should toggle ON or OFF, and OLED updates accordingly.

- Long press (hold for about 2 s): buzzer sounds, OLED shows “Long press detected”.

- Ensure no unintended toggles due to bounce or very short presses.

### 5. Summary

This project demonstrates how to differentiate between short and long button presses, taking different actions accordingly (LED toggle vs buzzer), and providing user feedback via an OLED display. It covers digital input reading, timing for press duration, output control (LED + buzzer), and display interaction in an embedded setup.

If you like, I can create a schematic diagram of your wiring and a formatted PDF documentation ready for submission. Would you like that?

##### You said:

bit long

###### ChatGPT said:

Here’s a short version of the documentation for your Wokwi project 👇

## Task B — Button Press Type Detection

Author: Malaika Batool
Reg No: 23-NTU-CS-1044 Date: 26-10-2025

### 1. Objective

Detect short and long button presses using an ESP32:

- Short press (<1.5 s): Toggle LED

- Long press (>1.5 s): Activate buzzer

- OLED shows press type and current state

### 2. Hardware Setup

| Component | Pin | Function |
| --- | --- | --- |
| Button | GPIO14 | Input (press detection) |
| LED | GPIO16 | Toggle on short press |
| Buzzer | GPIO25 | Beep on long press |
| OLED | SDA → GPIO21, SCL → GPIO22 | Display messages |

All buttons use INPUT_PULLUP mode.

### 3. Software Overview

- Uses Adafruit_SSD1306 and Adafruit_GFX for OLED.

- Detects press duration using millis().

- If <1500 ms: toggles LED and shows “Short Press”.

- If ≥1500 ms: sounds buzzer and shows “Long Press”.

- Includes debounce to prevent false readings.

### 4. Testing

✅ OLED shows “Ready…” at start
✅ Short press → LED toggles
✅ Long press → Buzzer sounds
✅ Messages update correctly

### 5. Summary

A simple project demonstrating button timing, digital I/O, buzzer control, and OLED feedback on ESP32.